ABV – Indian Institute Of Information technology and Management Gwalior, Madhya Pradesh

Microprocessor and Embedded System

Project Report

Design and Implementation of Real-Time Heart Rate, SpO₂ and Arrhythmia Detection System using Machine Learning

Group – 4 :

Submitted By :-

- Aditya Amitabh Yadav (2023BEE - 003)

- Jaisurya Vetrivel (2023BEE - 016)

- Madhav Gupta (2023BEE - 023)

- Mahendra Meena (2023BEE - 024)

Under the Guidance of :

Prof. Manisha Pattanaik

Department of Electrical and Electronics

Submission date :

April 11, 2026

Design and Implementation of Real-Time Heart Rate, SpO₂ and Arrhythmia Detection System using Machine Learning

Abstract

Cardiovascular diseases are one of the leading causes of death worldwide and often develop without noticeable early symptoms. Continuous monitoring of vital parameters such as heart rate and oxygen saturation (SpO2) is essential for early detection and prevention. However, existing monitoring systems are either expensive and non-portable (hospital equipment) or lack transparency and accuracy (commercial smartwatches).

This project presents a low-cost, portable, and intelligent health monitoring system based on Photoplethysmography (PPG). The system uses the MAX30102 sensor to capture physiological signals and processes them using a hybrid approach combining signal processing, rule-based clinical logic, and a lightweight 1D Convolutional Neural Network (CNN). The system performs real-time monitoring and displays results on a web dashboard.

The proposed solution achieves high accuracy while maintaining low computational cost, making it suitable for wearable and edge-device deployment.

Introduction

Cardiovascular diseases such as arrhythmia and sudden cardiac arrest are increasing rapidly across the globe. These conditions often develop silently, making early detection difficult without continuous monitoring.

Traditional hospital monitoring systems are highly accurate but expensive and non-portable, limiting their accessibility. On the other hand, wearable devices like smartwatches provide portability but often act as closed systems (“black boxes”) and lack access to raw biological signals for research and analysis.

Photoplethysmography (PPG) is a non-invasive optical technique that measures blood volume changes in tissues using light. It enables the measurement of heart rate and oxygen saturation using a simple single-point contact, such as a fingertip.

This project aims to bridge the gap between clinical-grade monitoring and consumer-level accessibility by developing a low-cost, transparent, and efficient monitoring system.

Literature Review

Several approaches exist for heart monitoring:

- ECG-based monitoring systems

- Wearable smart devices

- Hospital-grade monitoring equipment

- Machine learning-based arrhythmia detection

However, these approaches suffer from key limitations:

- High cost

- Lack of portability

- Complex hardware setup

- Limited accuracy in real-world conditions

Previous research has utilized datasets such as the MIT-BIH dataset for arrhythmia detection. However, most systems rely on ECG signals, which require multiple electrodes and are less convenient.

PPG-based systems provide a better alternative due to their simplicity, non-invasive nature, and ability to measure multiple parameters simultaneously.

Problem Statement

Current health monitoring systems face the following issues:

- Hospital systems are expensive and not portable

- Smartwatches lack transparency and raw data access

- Continuous monitoring is not affordable for many users

- Devices are not optimized for real-time edge processing

Therefore, there is a need for a low-cost, portable, and intelligent system capable of real-time monitoring and accurate diagnosis.

Proposed Methodology

The proposed system uses a hybrid approach combining signal processing, machine learning, and rule-based logic.

System Flow:

- PPG signal acquisition using MAX30102 sensor

- Data transmission via I2C protocol

- Buffering of incoming data

- Signal filtering using a bandpass filter

- Peak detection to identify heartbeats

- RR interval calculation

- Feature extraction (Mean RR, RMSSD, pNN50, etc.)

- Machine learning prediction using 1D CNN

- Rule-based validation using clinical thresholds

- Display of results on a web dashboard

System Design / Architecture

The system consists of three main components:

1. Hardware

- Sensor: MAX30102 (PPG sensor)

- Processor: Raspberry Pi 5

- Communication: I2C protocol

2. Software

- Signal processing using Python (SciPy)

- Filtering: Butterworth bandpass filter (0.5–4 Hz)

- Peak detection using derivative-based methods

3. AI Model

- 1D CNN for time-series classification

- Binary classification (Normal vs Arrhythmia)

- Lightweight model (2–3 million FLOPs)

Implementation Details

Signal Processing

- Filtering removes noise and isolates heart rate frequencies

- Peak detection identifies heartbeat locations

- RR intervals measure time between beats

Feature Extraction

- Mean RR interval

- RMSSD (Root Mean Square of Successive Differences)

- pNN50

- Standard deviation and variance

SpO2 Calculation

- Based on ratio of Red and IR light absorption

Machine Learning

- Dataset: UMass Simband dataset

- Model: 1D CNN

- Deployment: TensorFlow Lite on Raspberry Pi

Rule-Based Logic

- CV (Coefficient of Variation)

- RMSSD thresholds

- Classification:

  - Normal

  - Irregular

  - Arrhythmia

Results & Discussion

The system successfully performs real-time monitoring of heart rate and SpO2 with high stability.

Performance:

- Real-time BPM and SpO2 calculation

- Robust signal processing pipeline

- Efficient edge-device deployment

Accuracy:

- Machine Learning Model: 98% accuracy

- Rule-Based Method: 95% accuracy (resting conditions)

Comparison:

| Metric | Rule-Based | ML Model |
| --- | --- | --- |
| Accuracy (Rest) | 95% | 98% |
| Noise Handling | Poor (~40%) | Strong |
| Inference Time | <1 ms | <10 ms |

Observations:

- ML model performs better under motion noise

- Native PPG training reduces false alarms

- Hybrid system improves reliability

Novelty

- Use of native PPG data instead of ECG-derived signals

- Deployment of 1D CNN on edge device (Raspberry Pi 5)

- Hybrid system combining AI + clinical rules

- Real-time web dashboard integration

- Transparent system allowing access to raw data

Contributions

The project was carried out collaboratively by a team of four members, organized into two pairs to ensure efficient development and parallel progress across different components of the system.

Pair 1: Software, Web & Validation

Members: Madhav Gupta, Aditya Amitabh Yadav

This team focused on the software infrastructure and validation of the system:

- Developed the web-based dashboard using Flask and HTML/JavaScript for real-time visualization of health parameters.

- Implemented rule-based diagnostic algorithms using clinical metrics such as CV and RMSSD.

- Designed and executed system validation strategies, including testing with simulated and real signal conditions.

- Managed data logging, monitoring, and performance evaluation.

- Contributed to documentation and report preparation.

Pair 2: Hardware, Signal Processing & AI

Members: Jaisurya Vetrivel, Mahendra Meena

This team handled the hardware setup and core signal processing pipeline:

- Integrated the MAX30102 sensor with the Raspberry Pi using the I2C protocol.

- Developed the signal processing pipeline, including filtering, peak detection, and RR interval extraction.

- Implemented and optimized the 1D CNN model for arrhythmia detection.

- Converted and deployed the model using TensorFlow Lite for edge-device execution.

- Conducted real-time testing and ensured seamless interaction between hardware and software modules.

Conclusion

This project successfully developed a low-cost, portable, and accurate health monitoring system using PPG signals. The integration of machine learning and rule-based logic ensures both accuracy and reliability.

The system bridges the gap between clinical-grade monitoring and wearable devices by providing an open, efficient, and real-time solution.

Future Scope

- Improve motion artifact handling using advanced models

- Cloud integration for long-term monitoring

- Multi-sensor fusion (temperature, activity tracking)

- Expansion for multi-user healthcare systems

References

- UMass Simband Dataset https://github.com/Cassey2016/UMass_Simband_Dataset

- MAX30102 Datasheet (Analog Devices)

- AHA/ACC Clinical Guidelines

- Python, SciPy, TensorFlow Lite documentation

- Research papers on PPG and arrhythmia detection

- Multiclass arrhythmia classification using multimodal smartwatch photoplethysmography signals collected in real-life settings

- Atrial fibrillation Detection from Wrist photoplethysmography Signals Using Smartwatches

- Premature Atrial and Ventricular Contraction Detection Using Photoplethysmographic Data from a Smartwatch